Project Context Summary

# پروژه: Tag Core (SCADA/Historian Core)

## Tech Stack

- Language: C++ (Qt 5.14.2, MinGW 7.3.0 64-bit, Windows)

- Database: PostgreSQL + TimescaleDB (QPSQL driver)

- Build System: qmake (.pro)

- External libs: qmqtt (MQTT), open62541 (OPC UA - در حال build), libmodbus (not yet)

- QT modules: core sql network serialport qml

## معماری کلی

Config → bootstrap.json (فقط DB connection)

PostgreSQL (کل کانفیگ: tags, drivers, rules, settings)

DriverManager

├── SimulatorDriver (via Adapter)

├── ModbusTcpDriver (FC3/4/5/6/16, read+write, Batch Read)

├── ModbusRtuDriver (QSerialPort, CRC16, Batch Read)

├── MqttDriver (qmqtt, lambda signals)

└── [OpcUaDriver - TODO: نیاز به build open62541]

Data Flow:

Driver → tags/{id}/raw

→ FilterProcessor (Software Filter + Rounding)

→ tags/{id}/update

→ [CurrentStateWriter | StorageExceptionFilter | RuleEngine | RealtimeCache]

Storage:

tag_values_raw (Hypertable)

tag_current_state (UPSERT)

Continuous Aggregates: 1min/5min/1hour/1day

Compression + Retention Policies

ArchiveManager (CSV export هر 100MB)

alarms + alarm_events

archive_log

notification_log

## Feature های پیاده‌سازی شده

### Core

- ✅ TagBus (pub/sub با topic های raw/update/alarms)

- ✅ DbManager با migration خودکار

- ✅ ConfigLoader (bootstrap.json → DB seed)

- ✅ Models (TagDefinition, ThresholdRule, DriverDefinition, ComputedTag, ...)

- ✅ AppConfig از DB (system_settings)

### Processing Pipeline

- ✅ ScalingEngine (linear + min_max + reverseScale برای write)

- ✅ SoftwareFilterChain (none, moving_average, exponential_average, median, debounce, outlier_rejection, rate_limiter)

- ✅ StorageExceptionFilter (deadband + heartbeat + quality bypass)

- ✅ Rounding (engineering_decimals = 4)

- ✅ BatchHistorianWriter (transaction-based)

- ✅ CurrentStateWriter (batch upsert)

- ✅ DeadbandPolicy (global_min_deadband)

### Rule Engine (9 نوع آلارم)

- ✅ Threshold: HighHigh, High, Low, LowLow (با hysteresis + on/off delay)

- ✅ Range Violation

- ✅ Rate of Change

- ✅ Stuck Value

- ✅ Boolean State

- ✅ Bad Quality

- ✅ Severity (critical/high/medium/low/info)

- ✅ Alarm Events + Acknowledgment

### Notification

- ✅ NotificationManager با throttle

- ✅ WebhookNotifier (HTTP POST)

- ✅ LogNotifier

- ✅ Severity + AlarmType filtering

- ✅ notification_log table

### Computed Tags

- ✅ ComputedTagEngine با QJSEngine

- ✅ on_change + periodic modes

- ✅ Dependency tracking

### Historian

- ✅ Continuous Aggregates (1min/5min/1hour/1day)

- ✅ Compression + Retention Policies

- ✅ HistorianManager (query API)

- ✅ ArchiveManager (CSV export بر اساس حجم، 100MB threshold)

- ✅ archive_log table

### Drivers

- ✅ ITagDriver interface

- ✅ DriverFactory + DriverManager

- ✅ SimulatorDriver (sine/bad_quality/stuck)

- ✅ ModbusTcpDriver (FC3/4 read, FC5/6/16 write, debug mode, Batch Read)

- ✅ ModbusRtuDriver (QSerialPort, CRC16, Batch Read)

- ✅ MqttDriver (subscribe, JSON/raw payload, lambda signals)

- ✅ ModbusCardManager (Batch Read برای کارت‌های 16 سنسوره)

- 🚧 OpcUaDriver (نیاز به build open62541 با MinGW)

### Hardware Specific

- ✅ هر 16 سنسور روی یک کارت

- ✅ Card 0: Address 0-15, Card 1: Address 16-31, ...

- ✅ Batch Read: یک request برای هر کارت (16 registers)

- ✅ ModbusCardManager برای گروه‌بندی تگ‌ها بر اساس کارت

- ✅ address_config: {"card_index": 0, "sensor_index": 5, "data_type": "uint16"}

## ساختار فایل‌ها

src/

├── app/CoreApplication.{h,cpp}

├── core/{Models.h, ConfigLoader, ValueUtils.h}

├── tagbus/TagBus.{h,cpp}

├── storage/{DbManager, BatchHistorianWriter, CurrentStateWriter,

│ StorageExceptionFilter, HistorianManager, ArchiveManager}

├── filters/{ISoftwareFilter.h, SoftwareFilters.h, SoftwareFilterFactory, FilterProcessor}

├── scaling/ScalingEngine.h

├── ingestion/DeadbandFilter.h

├── realtime/RealtimeCache.{h,cpp}

├── rules/RuleEngine.{h,cpp}

├── notifications/{NotificationManager, WebhookNotifier}

├── computed/ComputedTagEngine.{h,cpp}

└── drivers/{ITagDriver.h, DriverManager, DriverFactory, ModbusTypes.h,

ModbusCardManager, SimulatorDriver[Adapter],

ModbusTcpDriver, ModbusRtuDriver, MqttDriver}

config/

├── bootstrap.json (فقط DB connection)

└── app.json (legacy, برای seed اولیه)

tag_core.pro (QT += core sql network serialport qml)

## نکات مهم حل‌شده

1. `offset` در PostgreSQL reserved است → ستون `scaling_offset`

2. `QVariant::toDouble(default)` در Qt 5.14 خطا می‌دهد → helper functions (variantToDouble, variantToString, ...)

3. `Qt::SkipEmptyParts` در Qt 5.14 وجود ندارد → `QString::SkipEmptyParts`

4. `QNetworkRequest request(QUrl(url))` → Most Vexing Parse → `QUrl requestUrl(url); QNetworkRequest request(requestUrl);`

5. `QMQTT::Client(m_host, ...)` → `QHostAddress::LocalHost` یا `QHostAddress(m_host)`

6. MqttDriver signals با lambda کار می‌کند، با member function نه (به دلیل multiple inheritance)

7. ArchiveManager نیاز به `#include <QSqlError>` دارد

8. `QVector` متد `dequeue` ندارد → استفاده از `QQueue`

9. ModbusRtuDriver نیاز به `expectedResponseSize` برای تشخیص فریم کامل RTU دارد

10. Modbus TCP/RTU هر دو از ModbusCardManager برای Batch Read استفاده می‌کنند

## مرحله بعدی: API Layer (REST + WebSocket)

### هدف

سرویس‌دهی به GUI های مستقل:

1. GUI آلارم‌های آنالوگ

2. GUI آلارم‌های دیجیتال

3. GUI گراف‌های آنلاین

4. GUI گزارش‌گیری

5. GUI مدیریت تگ‌ها

6. GUI صفحات آنلاین کاربر

7. کنترل پنل مرکزی

### معماری پیشنهادی

Tag Core (Backend)

├── REST API Server (HTTP)

│ ├── /api/v1/tags (CRUD)

│ ├── /api/v1/alarms (list, ack, shelve)

│ ├── /api/v1/historian (query)

│ ├── /api/v1/reports

│ ├── /api/v1/drivers

│ ├── /api/v1/config

│ └── /api/v1/users

│

├── WebSocket Server

│ ├── /ws/alarms/analog

│ ├── /ws/alarms/digital

│ ├── /ws/tags/batch

│ ├── /ws/dashboard/{id}

│ └── /ws/system/status

│

└── MQTT Publisher (optional)

### کتابخانه‌های مورد نیاز

- Qt Http Server یا QHttpServer (REST API)

- Qt WebSockets (WebSocket Server)

- qmqtt (موجود)

### ساختار کلاس‌ها

src/api/

├── ApiServer.{h,cpp} ← مدیریت کل API

├── RestApiHandler.{h,cpp} ← REST endpoints

├── WebSocketServer.{h,cpp} ← WebSocket server

├── WebSocketHandler.{h,cpp} ← مدیریت connections

└── ApiAuthenticator.{h,cpp} ← احراز هویت

### مراحل پیاده‌سازی

1. WebSocket Server (برای داده‌های real-time)

2. REST API Server (برای CRUD و Query)

3. Authentication و Authorization

4. Report Generation

5. Dashboard API

این context پروژه من است. می‌خواهم مرحله بعدی یعنی API Layer (REST + WebSocket) را شروع کنیم. از مرحله 1 (WebSocket Server) شروع کن.